PROJECT PROPOSAL

MLinAPP

## Project Title, Project Number and Project Owner

|  | Detection of Anomalous Behaviour in Industrial Robot – 2026/AM01 – Alessio Mascolini |
| --- | --- |

## Project Team

|  | Oliver Hole Ekelund (student ID), [Team member 2] (student ID), [Team member 3] (student ID), [Team member 4] (student ID) |
| --- | --- |

## Project Background

|  | Industrial robot arms like the KUKA manipulators used on modern production lines slowly drift away from their nominal behaviour as they age. Worn bearings, miscalibrated joints or stale configuration files can leave the robot still completing its programmed trajectories, but a bit too slowly or imprecisely. Symptoms like that hurt throughput and product quality long before they are large enough to trip a fixed-threshold alarm, which is exactly the kind of regime where a data-driven anomaly detector becomes useful. The project is part of the Machine Learning in Applications course at Politecnico di Torino, with Alessio Mascolini as the project owner. We are working on the KukaVelocityDataset, a multivariate time-series collection recorded on a KUKA arm. It comes in two splits. KukaNormal contains roughly 234k timesteps over 86 channels of nominal operation. KukaSlow contains roughly 41k timesteps of the same 32 actions executed at a reduced velocity, used here as a stand-in for the slow, imprecise motion that real-world degradation would produce. The setting has all the usual industrial-anomaly-detection difficulties at once. There are 86 input channels (electrical readings plus accelerometers, gyroscopes, quaternions and temperatures from seven IMUs), no labelled anomalies in the training distribution, and obvious temporal structure that a flat per-sample model would ignore. We have two goals. First, fit a model on the normal data alone that captures the operating envelope. Second, see whether adding an adversarial component to a standard autoencoder actually helps on this dataset, or whether the extra training cost buys nothing. |
| --- | --- |

## Project Scope

|  | In scope. We will deliver a reproducible time-series anomaly detection pipeline on the KukaVelocityDataset. The work breaks down as follows. We start by exploring and pre-processing the sensor data: windowing, normalisation, and a sanity pass over the 86 channels. We then implement a reconstruction-based autoencoder trained only on KukaNormal, and an adversarial autoencoder that adds a discriminator on the latent code (and possibly on the reconstruction itself) to regularise the learnt representation. From those two models we derive an anomaly score and a decision threshold using the reconstruction error, the discriminator signal, or some combination of the two. Finally we evaluate both models on the held-out normal split and on the full KukaSlow set with time-series-aware metrics. The comparison covers detection quality, but also the practical side: how long each model takes to train and how stable the training is. Out of scope. No new data is being recorded for this project, we are not designing sensors, and nothing will be pushed onto a real KUKA controller. We also stop short of localising or classifying specific failure modes; the output of the system is just a binary normal vs. slow decision. We also do not stray into other model families like supervised classifiers or large foundation models. The project specification asks for an AE/AAE comparison and that is what we deliver. Expected result. By the exam date we want a clean codebase for both models, a set of trained checkpoints, an experimental report, and a clear answer to the central question: does the adversarial component actually beat a plain autoencoder on this Kuka data, and is it worth the extra training cost? |
| --- | --- |

## High-Level Requirements

|  | Software stack. Python 3.11+ and PyTorch for everything model-related. NumPy and pandas to handle the .npy arrays, scikit-learn for preprocessing and the classical baselines, and Matplotlib / Seaborn for plots. Experiment tracking goes through Weights & Biases, with TensorBoard as a fallback if access becomes a problem. The code lives in a private GitHub repository. Reference repositories / literature. The Adversarial Autoencoder paper by Makhzani et al. (2015) is the model we are reproducing, together with several PyTorch implementations of it that we use as references. For evaluation we follow Kim et al. (2022), “Towards a Rigorous Evaluation of Time-series Anomaly Detection”. Open-source AE/AAE pipelines for industrial time series, in particular TranAD and USAD, are useful for code patterns and as comparison points, but they are not the main model we build. Computational resources. Early development and prototyping happens on team laptops and on free GPU services (Google Colab and Kaggle Notebooks). Once we move to longer sweeps and the final training runs we will need a real GPU, so we are requesting access to the PoliTO server for the team. If access is not granted, the fallback is paid Colab Pro sessions on a downsampled version of the dataset. |
| --- | --- |

## Implementation Plan

|  | 1. Data understanding and preprocessing. The dataset comes as three NumPy arrays: KukaNormal (~234k samples × 86 channels, all nominal), KukaSlow (~41k samples × 87 channels, all anomalous with the anomaly column set to 1), and KukaColumnNames. The 86 feature columns are: 1 action label, 8 robot-level electrical signals (apparent power, current, frequency, phase angle, power, power factor, reactive power, voltage), and 7 IMUs at 11 channels each (3 accelerometer, 3 gyroscope, 4 quaternion, 1 temperature). The preprocessing pass computes per-channel statistics, flags constant or near-constant channels (a quick look already shows that some gyroscope axes are flat), builds sliding-window sequences keyed by the action label, and standardises features using statistics fitted only on the normal training split. 2. Baseline autoencoder. The baseline is a reconstruction autoencoder trained only on KukaNormal. We start from a 1D-convolutional autoencoder over fixed-length windows, and if time permits we also try an LSTM or small Transformer encoder for comparison. Each window's reconstruction error is its anomaly score. To pick the decision threshold we hold out a slice of the normal data; we either target a fixed false-positive rate, or maximise F1 on a small mixed validation set if one is available. 3. Adversarial autoencoder. The AAE keeps the same encoder/decoder backbone as the baseline and adds a discriminator that pushes the latent code toward a chosen prior (a standard Gaussian, following Makhzani et al.). The training loss is a weighted sum of the reconstruction term and an adversarial term. Time permitting we also try the USAD-style two-decoder variant, which has worked well for time series in the literature and gives us a second adversarial baseline. 4. Evaluation protocol. Picking the right metrics is an explicit part of the examination. We follow Kim et al. (2022) and report both threshold-independent metrics (ROC-AUC, PR-AUC) and threshold-dependent ones (precision, recall, F1), each computed per window and again with the proper time-series adjustments. We deliberately avoid the well-known “point-adjust” inflation trick and use the stricter range-based / event-based variants instead. Evaluation runs on a held-out normal split together with the full KukaSlow set, and we also slice the results by action label to see whether some actions are easier or harder to flag. 5. Experiments and ablations. The headline experiment compares the AE and the AAE on the same splits, the same hyperparameters and the same seeds. Around it we run sensitivity studies on window length, latent dimension and the adversarial loss weight, plus a feature-group ablation (electrical-only, IMU-only, or all 86 channels) to see where the anomaly signal mostly lives. We also keep an eye on the adversarial training itself: whether it converges reliably across seeds or needs babysitting. |
| --- | --- |

## Tentative Timeline/Schedule

|  | We are submitting this proposal at the end of April 2026 and plan to wrap the project up in about 10 weeks, with the oral exam in early-to-mid July 2026. The schedule below is a first cut and we will revise it after the first week of work. Phase 1 – Setup & literature review (week 1, 28 Apr – 4 May 2026). Set up the repository, environments and shared workflow. Read the core papers on autoencoders, AAE and time-series anomaly detection (Makhzani et al. 2015, Kim et al. 2022, USAD, TranAD). Phase 2 – Data exploration and preprocessing (weeks 2–3, 5 – 18 May 2026). Per-channel statistics on KukaNormal vs. KukaSlow, look for degenerate channels, action-aware visualisation, design of the windowing and normalisation pipeline, and the train / validation / test splits. Phase 3 – Baseline AE (weeks 3–4, 12 – 25 May 2026). Implement, train and tune the reconstruction-based autoencoder; first end-to-end evaluation on KukaSlow. Phase 4 – AAE implementation (weeks 5–6, 26 May – 8 June 2026). Add the discriminator and the adversarial training loop, stabilise training, run the first head-to-head comparison against the AE baseline. Phase 5 – Experiments & ablations (weeks 7–8, 9 – 22 June 2026). Hyperparameter sweeps, feature-group and window-length ablations, multi-seed evaluation, and error analysis. Phase 6 – Report & presentation (weeks 9–10, 23 June – 5 July 2026). Write the report, prepare the slide deck, clean up the code, and push the final version to the private GitHub repository at least one week before the agreed exam date. Target exam date: first half of July 2026, to be agreed by email with the project owner and Prof. Di Cataldo. |
| --- | --- |

## Confidentiality Agreement

By participating in this project, each group member agrees that all shared data, developed code, reports, and related documentation are confidential. These materials may be used solely for the purpose of completing this course exam and for no other purpose.
Any associated repositories or materials must not be made public or shared outside the group without prior consent.

# APProval to proceed

We approve the project as described above, and authorize the team to proceed.

| Name | Date |
| --- | --- |